# LOST CREEK TOWER CHECK LIST

## Date	 9-24-08

#### Transferred Files	262-267

## Deleted Files 		254-261

|  | ### PRESSURE | ### LINE | ### S/N |
| --- | --- | --- | --- |
| ## HI Pure N2 | 1100 | 4 |  |
| 555 ppm CO2 | 1200 | 5 |  |
| 334 ppm CO2 | 800 | 4 |  |
| 442 ppm CO2 | 1200 | 5 |  |
| Ind N2 | 500 | 4 |  |

|  | ### FLOW |
| --- | --- |
| ## Profiler sample cell (51) | 0 |
| Flux N2 (43) | 53/43 |
| Nafion N2 (51) | 52 |
| Profilers Ref cell (43) | 56/43 |
| Backflow Pressure | 0 |

| #### Battery Voltage | #### 26.2 |
| --- | --- |
| Amp Hours | 210.8/7661.5 |
| Array/Load Current | 6.0/158 |
| Water/Snow Depth | Creek higher than normal |
| Weather | Partly Cloudy |

Comments:     Pump and LiCor off again.  Checked battery water level.  They seemed fine.  One battery cable was very corroded so I cleaned it, and another was very loose so I cleaned and tightened it.