Description of flux data for Willow Creek Tower in Wcreekyyy_flux.txt

Missing or invalid data is reported as -999.

Time is recorded at the beginning of average period.

The average interval is 30 minutes.

Note,

We believe that NEE data does not reflect stand fluxes when:

- u* is small (<0.175m/s); and

- winds are from the southeast (direction of nearby wetlands), and there is a temperature inversion at the canopy (i.e., a decoupling of the surface measurements).

The flag in Column 13 indicates flux measurement quality based on the environmental conditions above:

0 = acceptable measurement;

1 = unacceptable measurement based on condition (1); and

2 = unacceptable measurement based on condition (2).

| Column | Name | Description | Unit |
| --- | --- | --- | --- |
| 1 | Year | Year | Year |
| 2 | Month | Month | Month |
| 3 | Day | Day | Day |
| 4 | LST | Local Standard Time | Hour |
| 5 | Jday | Julian day | Day |
| 6 | Fjday | Fractional Julian day | Day |
| 7 | Sc_97ft | Rate of change of CO2 storage at 97 feet |  mol m-2s-1 |
| 8 | Fc_97ft | CO2 flux at 97 feet |  mol m-2s-1 |
| 9 | Nee_97ft | CO2 NEE at 97 feet=Sc_97 ft+Fc_97 ft |  mol m-2s-1 |
| 10 | LE_97ft | Latent heat flux at 97 feet | W m-2 |
| 11 | H_97ft | Sensible heat flux at 97 feet | W m-2 |
| 12 | Ustar30 | Friction velocity at 97 feet | m s-1 |
| 13 | Flag | The flag indicating the flux quality (see above) | 1, 2, or 3 |