Description of micrometeorology data for Willow Creek Tower in Wcreekyyyy_met.txt

Missing or invalid data is reported as -999. Time is recorded at the beginning of average period. The average interval is 30 minutes.

| Column | Name | Description | Unit |
| --- | --- | --- | --- |
| 1 | Year | Year | Year |
| 2 | Month | Month | Month |
| 3 | Day | Day | Day |
| 4 | LST | Local Standard Time | Hour |
| 5 | Jday | Julian day | Day |
| 6 | Fjday | Fractional Julian day | Day |
| 7 | CO2_2ft | Concentration of CO2 at 2 feet | ppm |
| 8 | CO2_5ft | Concentration of CO2 at 5 feet | ppm |
| 9 | CO2_10ft | Concentration of CO2 at 10 feet | ppm |
| 10 | CO2_25ft | Concentration of CO2 at 25 feet | ppm |
| 11 | CO2_45ft | Concentration of CO2 at 45 feet | ppm |
| 12 | CO2_70ft | Concentration of CO2 at 70 feet | ppm |
| 13 | CO2_97ft | Concentration of CO2 at 97 feet | ppm |
| 14 | Wsp_2m | Wind speed at 2 meters | m/s |
| 15 | Wsp_40ft | Wind speed at 40 feet | m/s |
| 16 | Wsp_80ft | Wind speed at 80 feet | m/s |
| 17 | Wsp_97ft | Wind speed at 97 feet | m/s |
| 18 | Wdir_2m | Wind direction at 2 meters | Deg |
| 19 | Wdir_40ft | Wind direction at 40 feet | Deg |
| 20 | Wdir_80ft | Wind direction at 80 feet | Deg |
| 21 | Wdir_97ft | Wind direction at 97 feet | Deg |
| 22 | Ta_25cm | Air temperature at 25 cm | C |
| 23 | Ta_50cm | Air temperature at 50 cm | C |
| 24 | Ta_75cm | Air temperature at 75 cm | C |
| 25 | Ta_1 m | Air temperature at 1 meter | C |
| 26 | Ta-2 m | Air temperature at 2 meters | C |
| 27 | Ta_25 ft | Air temperature at 25 feet | C |
| 28 | Ta_40 ft | Air temperature at 40 feet | C |
| 29 | Ta_60 ft | Air temperature at 60 feet | C |
| 30 | Ta_80 ft | Air temperature at 80 feet | C |
| 31 | Ta_97 ft | Air temperature at 97 feet | C |
| 32 | Tree_n | Tree temperature, north side | C |
| 33 | Tree_s | Tree temperature, south side | C |
| 34 | Mr_97 ft | H2O mixing ratio at 97 feet | g/kg |
| 35 | Rh_2m | Relative humidity at 2 meters | % |
| 36 | Rh_25 ft | Relative humidity at 25 feet | % |
| 37 | Rh_40 ft | Relative humidity at 40 feet | % |
| 38 | Rh_60 ft | Relative humidity at 60 feet | % |
| 39 | Rh_80 ft | Relative humidity at 80 feet | % |
| 40 | Ts_0 | Soil temperature on surface | C |
| 41 | Ts_5 cm | Soil temperature at 5cm below surface | C |
| 42 | Ts_10 cm | Soil temperature at 10 cm below surface | C |
| 43 | Ts_20 cm | Soil temperature at 20 cm below surface | C |
| 44 | Ts_50 cm | Soil temperature at 50 cm below surface | C |
| 45 | Ts_1 m | Soil temperature at 1 meter below surface | C |
| 46 | SWC_5 cm | Soil moisture content at 5 cm below surface | % by vol |
| 47 | SWC_10 cm | Soil moisture content at 10 cm below surface | % by vol |
| 48 | SWC_20 cm | Soil moisture content at 20 cm below surface | % by vol |
| 49 | SWC_50 cm | Soil moisture content at 50 cm below surface | % by vol |
| 50 | SWC_1 m | Soil moisture content at 1 meter below surface | % by vol |
| 51 | P_atm | Atmospheric pressure at 97 feet | Kpa |
| 52 | PPFD_2m | Photosynthetic Active radiation at 2 meters |  mol m-2 s-1 |
| 53 | PPFD_20 ft | Photosynthetic Active radiation at 20 feet |  mol m-2 s-1 |
| 54 | PPFD_40 ft | Photosynthetic Active radiation at 40 feet |  mol m-2 s-1 |
| 55 | PPFD_60 ft | Photosynthetic Active radiation at 60 feet |  mol m-2 s-1 |
| 56 | PPFD_97ft | Total incoming photosynthetic photon flux density at 97 feet |  mol m-2 s-1 |
| 57 | PPFD_dif | Diffuse incoming photosynthetic photon flux density at 97 feet |  mol m-2 s-1 |
| 58 | P | Precipitation | Mm |
| 59 | Solar_in | Incoming solar radiation at 97 feet | W m-2 |
| 60 | Solar_ref | Reflected solar radiation at 97 feet | W/m-2 |
| 61 | Ir_in | Incoming ir radiation at 97 feet | W/m-2 |
| 62 | Ir_emit | Emitted ir radiation at 97 feet | W/m-2 |
| 63 | Sol_stdev | Standard deviation of incoming solar radiation | W/m-2 |
| 64 | R_in | Incoming total radiation at 97 feet | W m-2 |
| 65 | R_net | Net radiation at 97 feet | W m-2 |
| 66 | Leaf-wet | Wetness indicator at 97 feet | Kohms |
| 67 | Net_raw | Net radiation above soil surface, no wind correction at 2m | W m-2 |
| 68 | Net_rad | Net radiation above soil surface, wind correction applied at 2m | W m-2 |
| 69 | G_75 mm | Soil heat flux at 75 mm below surface | W m-2 |